# Section 1: IIR Filter Design

- Distinguish between Butterworth and Chebyshev filters.

- What is prewarping? Why is it needed?

- Distinguish between FIR and IIR filters.

- Give the bilinear transformation equation.

- What is warping effect? What is its effect on magnitude and phase response?

- What are the advantages and disadvantages of bilinear transformation?

- Give the expression for the location of poles of a normalized Butterworth filter.

- What are the parameters (specifications) of a Chebyshev filter?

- Why is the impulse invariance method not preferred for filters other than lowpass?

- Write the transfer function of a first-order Butterworth lowpass filter.

- Give the equation for converting a normalized LPF into a BPF.

- Distinguish between the frequency response of Chebyshev Type I and Type II filters.

- What is meant by the impulse invariant method of designing an IIR filter?

- What are the different structures for realization of IIR systems?

- What is the transposition theorem and transposed structure?

- Draw the parallel-form structure of an IIR filter.

- What is the most general form of an IIR filter?

- How many additions, multiplications, and memory locations are required to realize H(z) with M zeros and N poles in Direct Form I and Direct Form II?

- What is the main drawback of the impulse invariant method, and how does bilinear transformation overcome it?

- Give the advantage of Direct Form II realization.

- Differentiate between recursive and non-recursive filters.

# Section 2:  FIR Filter Design

- What are the desirable and undesirable features of an FIR filter?

- Discuss the stability of FIR filters.

- What are the main advantages of FIR over IIR?

- What condition must h(n) satisfy for a linear-phase FIR filter?

- What is Gibbs phenomenon?

- Show that the filter with h(n)={−1,0,1} is a linear-phase filter.

- Explain the procedure for designing FIR filters using windows.

- What are the desirable characteristics of windows?

- Draw the frequency response of an N-point rectangular window.

- Give the equations specifying the Hanning and Blackman windows.

- What is the necessary and sufficient condition for linear-phase characteristics in an FIR filter?

- Give the equation specifying the Kaiser window.

- Compare rectangular and Hanning window functions.

- Briefly explain the frequency-sampling method of filter design.

- Compare frequency-sampling and windowing methods of filter design.

- List the different forms of structural realization for an FIR system.

- Define backward and forward prediction in an FIR lattice filter.

# Section 3: Calculation Practice — Difference Equations & Realizations (Li Tan Ch. 6)

These are the exact source problems your test questions were drawn from. Practice all, especially the last two (direct-form realization) — that's the format used on your recent test.

- y(n) = x(n−1) − 0.75y(n−1) − 0.125y(n−2): (a) calculate the system response y(n) for n=0,1,2,3,4 with input x(n)=(0.5)ⁿu(n) and initial conditions x(−1)=−1, y(−2)=2, y(−1)=1; (b) repeat with zero initial conditions.

- y(n) = 0.5x(n) + 0.5x(n−1): (a) find H(z); (b) find impulse response for x(n)=4δ(n); (c) find step response for x(n)=10u(n).

- y(n) = x(n) − 0.5y(n−1): (a) find H(z); (b) find impulse response for x(n)=δ(n); (c) find step response for x(n)=u(n).

- y(n) = x(n) − 0.25x(n−2) − 1.1y(n−1) − 0.28y(n−2): find the transfer function H(z), the denominator polynomial A(z), and the numerator polynomial B(z).

- y(n) = x(n) − 0.3x(n−1) + 0.28x(n−2): find the transfer function H(z), A(z), and B(z).

- Convert each of the following transfer functions into its difference equation:

(a)  H(z)=z²-0.25z²+1.1z+0.18

(b)  H(z)=z²-0.1z+0.3z³

- Convert the following transfer function into its pole-zero form:

H(z)=1-0.16z⁻²1+0.7z⁻¹+0.1z⁻²

- A transfer function depicting a digital system is given below. (a) Determine the impulse response h(n) and step response; (b) determine the system response y(n) if the input is x(n)=(0.25)ⁿu(n).

H(z)=10(z+1)z+0.75

- Given each of the following transfer functions that describe digital system transfer functions, sketch the z-plane pole-zero plot and determine the stability for each digital system.

(a)  H(z)=z-0.5(z+0.25)(z²+z+0.8)

(b)  H(z)=z²+0.25(z-0.5)(z²+4z+7)

(c)  H(z)=z+0.95(z+0.2)(z²+1.414z+1)

(d)  H(z)=z²+z+0.25(z-1)(z+1)²(z-0.36)

- Given a digital system with a sampling rate of 8,000 Hz, y(n)=0.5x(n)+0.5x(n−2): (a) determine the frequency response; (b) calculate and plot the magnitude and phase frequency responses; (c) determine the filter type.

- Given a digital system with a sampling rate of 8,000 Hz, y(n)=x(n)−0.5y(n−2): (a) determine the frequency response; (b) calculate and plot the magnitude and phase frequency responses; (c) determine the filter type.

- y(n)=x(n)−2cos(α)x(n−1)+x(n−2)+2γcos(α)−γ², where γ=0.8 and α=60°: (a) find H(z); (b) plot poles and zeros with the unit circle; (c) determine stability from the pole-zero plot; (d) calculate the magnitude response; (e) calculate the phase response.

- For each difference equation — (a) y(n)=0.5x(n)+0.5x(n−1); (b) y(n)=0.5x(n)−0.5x(n−1); (c) y(n)=0.5x(n)+0.5x(n−2); (d) y(n)=0.5x(n)−0.5x(n−2) — 1) find H(z); 2) calculate the magnitude response; 3) specify the filter type.

- y(n)=0.5x(n)+0.2y(n−1), y(−1)=0: (a) find H(z); (b) find the system response y(n) for input x(n)=(0.5)ⁿu(n).

- y(n)=0.5x(n)−0.7y(n−1)−0.1y(n−2), zero initial conditions: (a) find H(z); (b) find the unit step response.

- Given the following first-order IIR system, realize H(z) and develop the difference equations using (a) direct-form I; (b) direct-form II.

H(z)=1+2z⁻¹1-0.5z⁻¹

- Given the following filter, realize H(z) and develop the difference equations using (a) direct-form I; (b) direct-form II.

H(z)=1-0.9z⁻¹-0.1z⁻²1+0.3z⁻¹-0.04z⁻²

Worked reference example (Li Tan, Example 6.13): Given the transfer function below, perform the filter realization and write the difference equation using: (1) direct form I and direct form II; (2) cascade form via first-order sections; (3) parallel form via first-order sections.

H(z)=0.5-0.5z⁻²1+1.3z⁻¹+0.36z⁻²